** NOTE TO SPECIFIER ** LINCORA: Premium Standard and Custom Lockers

This section is based on the products of LINCORA, which is located at:

6265 Notre-Dame St. E.

Montreal, QC H1N 2E9

Toll Free: 800-564-9001

Phone: 514-253-5700

Email: info@lincora.com

Web: https://www.lincora.com

[Click Here] for additional information.

Our slogan says it all: Imagine your Locker™

Founded in 1975, LINCORA manufactures durable premium all-welded standard and custom lockers. Premium means, among other things, that it is common to see 30- or 35-year-old LINCORA lockers that are still structurally efficient when adapted to their environment and users.

LINCORA's clients value three main factors: durability, customization, and user satisfaction.

To this aim, LINCORA has created a powerful Configurator that allows you to:

1. Imagine and create a locker that will meet all your user's storage requirements.

2. Adapt the locker to your space and not the other way around

3. Adjust the dimensions of your lockers during their construction if needed

PS: Width, depth and height are customizable within 1/8 inches (3 mm).

The fact that LINCORA has automated the customization of its lockers allows us to thrive on complex projects and meet the needs of the following clientele: public safety, institutional, private sector including manufacturing and distribution, health care, education, and mining.

GENERAL

SECTION INCLUDES

** NOTE TO SPECIFIER ** Delete items below not required for project.

Welded metal lockers.

Changing room benches.

ADA compliant.

RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

Section 06100 - Rough Carpentry for blocking in frame walls required to anchor casework.

Section 09110 - Non-Load-Bearing Wall Framing for reinforcements in metal-framed partitions required to anchor casework.

REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not actually required by the text of the edited section.

Americans with Disabilities Act (ADA):

ADA AG - Americans with Disabilities Act, Accessibility Guidelines.

ASTM International (ASTM):

ASTM A366 - Standard Specification for Commercial Steel (CS) Sheet, Carbon (0.15 Maximum Percent) Cold-Rolled).

ASTM A653 - Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process.

SUBMITTALS

Submit under provisions of Section 01300.

Product Data:

Manufacturer's data sheets on each product to be used.

Preparation instructions and recommendations.

Storage and handling requirements and recommendations.

Typical installation methods.

** NOTE TO SPECIFIER ** Delete if not applicable to product type.

Verification Samples: Two representative units of each type, size, pattern, and color.

Shop Drawings: Include details of materials, construction, and finish. Include relationship with adjacent construction.

Verified Installation Field Dimensions: Drawings with the actual dimensions of the areas receiving the lockers but be submitted to the manufacturer prior to fabrication of the lockers and accessory equipment.

QUALITY ASSURANCE

Manufacturer Qualifications: Company specializing in manufacturing products specified in this section with a minimum of five years documented experience.

Installer Qualifications: Company specializing in performing work of this section with a minimum of two years documented experience with projects of similar scope and complexity.

Source Limitations: Provide each type of product from a single manufacturing source to ensure uniformity.

** NOTE TO SPECIFIER ** Include mock-up if the project size or quality warrant the expense. The following is one example of how a mock-up on might be specified. When deciding on the extent of the mock-up, consider all the major different types of work on the project.

Mock-Up: Construct a mock-up with actual materials in sufficient time for Architect’s review and to not delay construction progress. Locate mock-up as acceptable to Architect and provide temporary foundations and support.

The Intent of a mock-up is to demonstrate quality of workmanship and visual appearance.

If mock-up is not acceptable, rebuild mock-up until satisfactory results are achieved.

Retain mock-up during construction as a standard for comparison with completed work.

Do not alter or remove mock-up until work is completed or removal is authorized.

PRE-INSTALLATION CONFERENCE

Convene a conference approximately two weeks before scheduled commencement of the Work. Attendees shall include Architect, Contractor and trades involved. Agenda shall include schedule, responsibilities, critical path items and approvals.

DELIVERY, STORAGE, AND HANDLING

Store and handle in strict compliance with manufacturer's written instructions and recommendations.

Protect from damage due to weather, excessive temperature, and construction operations.

Store lockers in a manner that protects them from marks, scratches, and scuffs.

PROJECT CONDITIONS

Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's recommended limits.

WARRANTY

Manufacturer’s standard limited warranty unless indicated otherwise.

Lockers are warranted against defects in quality of materials and workmanship (including finish) for a period of 5 years from the date of final acceptance of the work.

No warranty on lockers that are placed in or close to a wet environment (such as a pool, waterfront, etc.) or nearby chemical products.

PRODUCTS

MANUFACTURERS

Acceptable Manufacturer: LINCORA, which is located at: 6265 Notre-Dame St. E.; Montreal, QC H1N 2E9; Toll Free: 1-800-564-9001; Phone: 514-253-5700; Email: info@lincora.com; Web: https://www.lincora.com

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs: coordinate with requirements of Division 1 section on product options and substitutions.

Substitutions: Not permitted.

Requests for substitutions will be considered in accordance with provisions of Section 01600.

PERFORMANCE AND DESIGN REQUIREMENTS

Verified Installation Field Dimensions: Drawings with the actual dimensions of the areas receiving the lockers must be submitted to the manufacturer prior to fabrication of the lockers and accessory equipment.

** NOTE TO SPECIFIER ** Delete article if not required or delete options not required.

ALL WELDED DOORLESS LOCKERS – SERIES 51

Basis of Design: Standard doorless   Locker 51 Series by LINCORA. All welded. No perforations on panels for assembly. Non-riveted.

** NOTE TO SPECIFIER ** Delete model, width, depth, and height options not required.

Model: Standard.

Model: Heavy Duty.

Model: Ultra Heavy Duty.

Tier: Single.

Width Range: Minimum: 9 inches (229 mm). Maximum: 48 inches (1219 mm). Increments: 1/8 inch (3 mm).

Depth Range: Minimum: 12 inches (305 mm). Maximum: 28 inches (711 mm). Increments: 1/8 inch (3 mm).

Height Range: Minimum: 30 inches (762 mm). Maximum: 90 inches (2286 mm). Increments: 1/8 inch (3 mm).

** NOTE TO SPECIFIER ** 46-inch height is only available as a single tier unit. Delete dimensions option not required.

Dimensions (WxDxH): ___ x ___ x ___ inches (___ x ___ x ___ mm).

Dimensions: As detailed in the Drawings.

Locker Body Construction:

Material: Premium quality cold rolled standard steel ASTM A366. No surface imperfections.

Frame: 16 ga (1.52 mm) sheet steel. A steel strip folded to form a 90-degree triple fold edge. The four corners are assembled by means of spot welds.

Bottom: Galvanneal sheet steel, A653 CS TY B.

** NOTE TO SPECIFIER ** Delete sheet steel material options not required. 22 ga applies to Standard Duty. 20 ga applies to Heavy Duty. 16 ga applies to Ultra Heavy Duty.

Sheet Steel Material: 22 ga (0.76 mm).

Sheet Steel Material: 20 ga (0.91 mm).

Sheet Steel Material: 16 ga (1.52 mm).

Sloped and perforated for drainage at exterior of recessed base.

Lateral and Back Flanges: Bent 90-degrees downward.

Front Flange: Double layered. Equivalent to a 16 ga (1.52 mm) lower frame.

The bottom is welded to the body.

Top: Lateral and back flanges bent 90-degrees downward and welded to the body.

** NOTE TO SPECIFIER ** Delete sheet steel material options not required. 22 ga applies to Standard Duty. 20 ga applies to Heavy Duty. 16 ga applies to Ultra Heavy Duty.

Sheet Steel Material: 22 ga (0.76 mm).

Sheet Steel Material: 20 ga (0.91 mm).

Sheet Steel Material: 16 ga (1.52 mm).

Front Flange: Double layered. Equivalent to equal a 16 ga (1.52 mm) upper frame.

** NOTE TO SPECIFIER ** Delete back and sides options not required. 22 ga applies to Standard Duty units. 20 ga applies to Heavy Duty units. 16ga applies to Ultra Heavy Duty units.

Back and Sides: 22 ga (0.76 mm) metal sheet.

Back and Sides: 20 ga (0.91 mm) metal sheet.

Back and Sides: 16 ga (1.52 mm) metal sheet.

Shelves:

Top Shelf: Sheet steel. 3 front folds, the third is flattened to eliminate sharp edge. Location: Welded at a minimum of 5 inches (127 mm) and a maximum of 45 inches (1143 mm) from the top.

** NOTE TO SPECIFIER ** Delete sheet steel material options not required. 22 ga applies to Standard Duty. 20 ga applies to Heavy Duty. 16 ga applies to Ultra Heavy Duty.

Sheet Steel Material: 22 ga (0.76 mm).

Sheet Steel Material: 20 ga (0.91 mm).

Sheet Steel Material: 16 ga (1.52 mm).

** NOTE TO SPECIFIER ** Second shelf is optional. Delete if not required. Different number of shelves and heights available on request.

Second Shelf: 8.5 inches (216 mm) below top shelf.

Second Shelf: 10 inches (254 mm) from bottom of locker.

** NOTE TO SPECIFIER ** Delete coat hooks option not required.

Coat Hooks: Flat steel 1/2 inch (13 mm) by 1/8 inch (3 mm) welded on plates which are then spot-welded to the sides.

Coat Hooks: Ball. Simple or double.

Hook Quantity: 3.

** NOTE TO SPECIFIER ** Delete accessory options not required.

Accessories and Options:

ADA Compliance:

Accessibility: One extra, adjustable shelf must be installed at a minimum height of 15 inches (381 mm) and a maximum of 48 inches (1220 mm) above the finish floor or ground space.

Convenience: Handle at a maximum height of 45 inches (1143 mm) above the finish floor or ground space.

Locks: Must open and close with one hand with no tight grasping, pinching, or twisting of the wrist.

Maximum Opening Pressure: 5 lbs force (22.2 N).

Recessed Base: 18 ga (1.21 mm) galvanized steel, ASTM A653 / A653M G30.

Finish: Black.

Finish: Same finish as locker.

Height: 2 inches (51 mm). Maximum: 30 inches (762 mm).

Recessed: Minimum: 3 inches (76 mm). Max: 6 inches (152 mm).

Sloped Top: 20 ga (0.91 mm) sheet metal.

Integrated and welded to the locker.

Installed on site.

Minimum height 3 inches (76 mm) Maximum 6 inches (152 mm)

Locker: All Galvanneal steel, A653 CS TY B, construction.

Number Plates: Black plastic.

Number Plates: Aluminum.

Ball Hooks: Simple or double.

Flat Hooks: Simple or double.

Finishing box end panel.

Recessed molding.

Benches:

** NOTE TO SPECIFIER ** Delete benches options not required.

Metal Legs Finish: Black.

Metal Legs Finish: Matching locker finish.

Metal Leg Shape: Square.

Metal Leg Shape: Round.

Bench Material: Oak.

Bench Material: Maple.

Bench Thickness: 1.25 inches (32 mm).

Width: 12 inches (305 mm).

Length: 36 to 96 inches (914 to 2438 mm).

Integrated bench and leg pedestal.

Finishes:

Preparation: Polish steel until imperfections affecting appearance and paint application are removed. Clean steel and protect against corrosion with a phosphate treatment.

** NOTE TO SPECIFIER ** Delete finish option not required.

Finish: Powder coated.

Dry Thickness Exposed Surfaces: Minimum 1 mil (0.025 mm).

Dry Thickness Other Surfaces: Minimum 0.6 mil (0.015 mm).

** NOTE TO SPECIFIER ** RAL, SICO, Benjamin Moore, Sherwin Williams, standard colors from other locker manufacturers or standard colors below. Delete color options not required.

Color: ________; custom.

Color: 9011 – White.

Color: 9005 – Platinum Grey.

Color: 9008 – Nevada Beige.

Color: 9012 – Aluminum Grey.

Color: 9014 – Medium Grey.

Color: 9070 – Pearl Grey.

Color: 9064 – Dark Grey.

Color: 9067 – Black.

Color: 9110 – Red.

Color: 9049 – Dark Blue.

Color: 9035 – Ocean Blue.

Color: 9036 – Azure Blue.

EXECUTION

EXAMINATION

Do not begin installation until the substrates have been properly constructed and prepared.

If substrate preparation is the responsibility of another installer, notify Architect in writing of unsatisfactory preparation before proceeding.

Verify field dimensions match data that was sent to Manufacturer for fabrication of lockers.

If dimensions do not match, notify the Architect in writing before proceeding.

PREPARATION

Clean surfaces thoroughly prior to installation.

Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

INSTALLATION

Install in accordance with manufacturer's instructions, approved submittals, and in proper relationship with adjacent construction.

Secure lockers to the cleats and nailing strips.

Install number plates and locking devices.

Optional installations:

Install wall trim around the recessed locker blocks.

Install filler panels (false fronts) where indicated and where there are obstacles.

Install finished bottom and end panels on all sides.

ADJUSTMENT

Adjust the lockers and their components to work properly, in accordance with the manufacturer's written instructions.

Precisely adjust and lubricate moving parts for smooth operation.

CLEANING AND PROTECTION

Cleaning during work: Perform cleaning work in accordance with the requirements of the General and Special Conditions.

Leave the premises clean at the end of each working day.

Clean surfaces with a damp cloth and an approved non-abrasive cleaning product in accordance with manufacturer's instructions.

Final Cleaning: Remove excess materials, waste, tools, and equipment from the job site in accordance with the requirements of the general and special conditions.

END OF SECTION